
DIVISION 12 00 00

FURNISHINGS
SECTION 12 93 00 

SITE FURNISHINGS

SECTION 12 93 43 

SITE SEATING AND TABLES

SECTION 12 93 43.13 

SITE SEATING

LAMINAR SEATING SYSTEM

PART 1 GENERAL
1.01 Summary
A. Section Includes:
1. 2 seats, backed bench.
2. 3 seats, backed bench.
3. 2 seats, backless bench.
4. 3 seats, backless bench.
1.02 Quality Assurance and References
A. Manufacturer qualifications
1. More than 60 years of experience designing and manufacturing metal structures.

2. In-house design, prototyping and perforated sheet production.
3. In-house assembly and quality control.
4. Provide reference list of customers using site seating fabricated by the manufacturer.

1.03 Submittals

A. Product data
1. Manufacturer’s technical data sheets.

2. Manufacturer’s installation instructions.

3. Manufacturer’s maintenance instructions.

4. Shop drawings.

5. Powder-coat finish and color samples for approval.

1.04 Delivery, Storage and Handling

A. Products must be handled in accordance with manufacturer’s maintenance instructions.

B. Products must be stored in manufacturer’s original packaging until ready for installation.

C. Protect materials and finish during storage and installation.

1.05 Warranty

A. Manufacturer’s standard warranty. 

1. Warranty terms: products are warranted against defects in material or workmanship for three years from the date of shipment. The warranty does not apply to damage resulting from vandalism, neglect, accident, misuse, tampering, or modification. 

2. A NOMEN USA service representative will inspect product to determine method of resolution.  Resolution options include repairing, replacing, or refunding the purchase price of any items found defective.

3. Scratches, nicks, and dents not reported prior to installation shall be considered normal wear and tear, and are not the responsibility of the manufacturer.

PART 2 PRODUCTS
2.01 Manufacturer

A. Basis of design: LAMINAR bench by NOMEN USA

1. Manufacturer Contact:
NOMEN USA
PO Box 7247
Fishers, IN 46038

Phone: 1 (800) 511-2616
Fax: 1 (317) 845-7502 

Email: sales@nomendesign.com

Website: www

HYPERLINK "http://www.nomendesign.com"
.

HYPERLINK "http://www.nomendesign.com"
nomendesign

HYPERLINK "http://www.nomendesign.com"
.

HYPERLINK "http://www.nomendesign.com"
com
2.02 Materials
A. Seats (Choose one seat type):
1. Backless seat (Choose one material type):
a. Stainless steel ANSI 304
b. Cold rolled carbon steel
2. Backed seat (Choose one material type):
a. Stainless steel ANSI 304
b. Cold rolled carbon steel
B. Mechanical Fasteners

a. Stainless steel ANSI 304
2.03 Finishes 
A. Seats and backs (Choose one below):
1. Stainless steel ANSI 304 (Choose one below):
a. Brushed
b. Shot blasted
2. Cold rolled carbon steel:

a. Undercoat: electro-coated with a corrosion resistant epoxy.
b. Topcoat: thermosetting polyester powder coat.

1) Color (Choose one below):
1. Black #001
2. Gray #002
3. White #003

4. Red #004

5. Orange #005

6. Yellow #006
7. Green #007

8. Blue #008
9. Custom color (Please contact Nomen USA for available options).
2.04 Dimensions (Choose from below)
A. 2 seats, backed bench:
Overall: 42.70" extended x 23.60" deep x 31.06" high
B. 3 seats, backed bench:
Overall: 62.40” extended x 23.60" deep x 31.06" high
C. 2 seats, backless bench:
Overall: 42.70" extended x 19.90" deep x 18.24" high

D. 3 seat, backless bench:
Overall: 62.40" extended x 19.90" deep x 18.24" high
2.05 Mounting
A. Standard surface mounting: The bench is attached with screws to threaded anchors installed in substrates. 

PART 3
 EXECUTION

3.01 Examination

A. Verify that sub-surfaces are level, stable, and adequate of supporting the weight of the product.

B. Confirm that mounting surface has been adequately prepared to securely anchor those products that will be surface mounted.

C. Architect must be notified if substrate conditions will adversely affect installation or use.

3.02 Installation

A. Install in accordance with manufacturer’s installation instructions.

B. Install all products level.

3.03 Cleaning
A. Clean all Nomen products prior to substantial completion with non-abrasive, mild soap and water.

3.04 Protection

A.  Protect all Nomen products to prevent any damage until the time of substantial completion.         

END OF SECTION

Page 1 of 3 | Rev. 05-09-14

